# SECTION 1 — GENERAL

## 1.1 Module Introduction

The Components Sub Module forms the foundational equipment register of the PMS.
It contains the full hierarchical structure of all machinery, equipment, and systems onboard a vessel, organized strictly as per SFI Codes.

This module serves as the master reference for the entire PMS and directly drives:

- Job scheduling

- Work Order generation

- Running Hours tracking

- Spare parts association

- Equipment documentation

- Class & statutory requirements

- PMS modifications

- Vessel-specific maintenance history

All other modules—such as Running Hours, Work Orders, Spares, Modify PMS, Reports—depend on the data stored and maintained here.

## 1.2 Purpose of This Module

This module is designed to:

- Provide a complete and accurate component hierarchy of a vessel using SFI classification.

- Store equipment-specific technical details, including Maker, Model, Serial No., Installation Date, and Criticality.

- Maintain a consistent Fleet Equipment Code (FEC) that standardizes equipment across vessels using the same model.

- Link all maintenance jobs, frequencies, and scheduling logic to the correct component.

- Serve as the main location where users view:

  - Component Information

  - Running Hours

  - Jobs

  - Maintenance History

  - Spares

  - Manuals & Drawings

  - Classification & Regulatory Data

- Provide controlled pathways for modifications through the Modify PMS module.

- Ensure all component data remains accurate, consistent, traceable, and vessel-specific.

## 1.3 Key Design Principles

- Strict SFI Structure

  - The complete hierarchy is strictly based on SFI codes.

  - Top-level SFI groups (1–8) cannot be changed.

- Consistent Fleet Equipment Code

  - FEC is generated based on Maker Code + Model Code.

  - Typically 10 characters (but not a strict limit).

  - Assigned only by the PMS software provider.

- Vessel-Specific Implementation

  - Each vessel has its own component register.

  - Same FEC across vessels; Component Codes differ per vessel.

- Traceability & Safety

  - All maintenance history is immutable.

  - Running Hours changes are delta-based and fully logged.

  - Criticality is controlled by company technical staff.

- Controlled Editing

  - Ship staff cannot modify component structure directly.

  - All structural changes go through Modify PMS for approval.

## 1.4 Module Dependencies (Functional Relationships)

The Components Module interacts with:

1. Running Hours Module

- Tracks current RH for parent components.

- Delta RH updates propagate to all child components.

- RH-based jobs depend entirely on RH module values.

2. Work Orders Module

- Calendar-based and RH-based jobs originate from Components.

- Work Orders feed Maintenance History in this module.

3. Spares Module

- Vessel-specific spares linked to each component appear in Section E.

- Fleet/company spares templates are not shown here.

4. Modify PMS Module

- Ship proposes changes (jobs, frequency, components, details).

- Office approves → changes reflect in Components Module.

5. Admin Module

Used for:

- Excel uploads of Component Registers

- Job Templates

- Spare Masters

- Maker/Model Masters

- Document Types

- Class Survey Templates

- Running Hours lead-time settings

6. Document Library

- Central storage for manuals, drawings, and regulatory files.

- Ship users get view-only access.

## 1.5 User Personas

1. PMS Software Provider (Admin)

- Full access to add/edit/delete fields, labels, templates.

- Upload complete PMS structures via Excel.

- Modify SFI hierarchy if needed (top-level groups remain fixed).

2. Company Superintendent / Technical Office

- Assigns Criticality.

- Enters Class/Regulatory details.

- Reviews & approves PMS modification requests.

- Controls job templates and vessel-specific customizations.

3. Ship Staff

- Can view the full Equipment Register.

- Can update Running Hours (through RH module).

- Can view all linked jobs, spares, manuals, and history.

- Cannot modify component details directly.

- Can only propose changes through Modify PMS.

4. Auditors / Class Surveyors

- View-only access for compliance and inspection purposes.

## 1.6 SFI Code Framework in PMS

The PMS uses the SFI Classification System for equipment breakdown.

Main Groups (Fixed)

1 – Ship General

2 – Hull

3 – Equipment for Cargo

4 – Ship’s Equipment

5 – Equipment for Crew & Passengers

6 – Machinery Main Components

7 – Systems for Machinery Main Components

8 – Ship Common Systems

## 1.7 Fleet Equipment Code (FEC)

A unique ID assigned by PMS Software Provider.

Key Points

- Typically 10 characters (not strictly enforced).

- Generated from Maker Code + Model Code.

- Identifies equipment type across the entire fleet.

- Used to link:

  - Jobs

  - Spares

  - Manuals

  - Templates

FEC cannot be modified by ship or company.

## 1.8 Component Criticality

Critical/Non-critical is assigned by the company superintendent.

Impact of Critical = YES

- Component appears in Critical filter.

- Jobs attached to component inherit critical status.

- Critical spares can be linked under Section E.

- Reports classify equipment as critical.

## 1.9 Running Hours Framework

Parent → Child Propagation (Delta Logic)

- RH entered in RH Module for parents.

- PMS calculates change (delta).

- Delta is added automatically to all child components.

- If child component was replaced → RH starts from zero, then delta added.

Logging

- Every RH update must be recorded with:

  - Timestamp

  - User

  - Delta value

  - New RH value

Editable here via pencil icon, but still logged.

## 1.10 Maintenance Job Logic

Sources of Jobs

- Admin Excel upload

- SFI-based job grouping

- Manual “Add Job” (office only)

Job Types

- Calendar-based

- Running-Hour based

Work Order Generation Rules

- Calendar jobs → based on date intervals

- RH jobs → based on Delta RH

- Lead-time is configurable in Admin module

- Ship staff cannot edit job details here

## 1.11 Manuals, Drawings & Documentation

- Central repository

- Uploaded by company or PMS admin

- Ship can view only with embedded PDF viewer

- Office can download files

- Multiple files may exist per component

## 1.12 Class & Regulatory Requirements

- Multiple survey rows allowed per component

- Supports Annual, Intermediate, Special, Damage Surveys

- Certificate No., Survey Type, Dates captured

- Can be uploaded by Excel or entered manually

- Office can update, ship cannot edit

## 1.13 Requisitions (Future Integration)

- Placeholder section

- Pane collapsed by default

- No user actions required

- Will connect to Purchase module in future

# SECTION 2 – UI STRUCTURE (COMPONENTS MODULE)

This section defines how the Components screen is laid out visually and how the main UI elements behave. It is written to be used directly as a UI/functional spec for PMS.

## 2.1 Overall Screen Layout

When the user clicks on PMS → Components in the top bar, the Components screen opens.

The screen is divided into three main zones:

- Top Navigation Bar (common across PMS)

- Left Navigation Sidebar (module navigation)

- Main Content Area

  - Top Filter Bar

  - Left Component Tree Pane

  - Right Component Details Pane

The layout is two-column in the main content area:

- Left column: Component Tree (SFI-based equipment hierarchy).

- Right column: Component details (Sections A–H) or “Select a component to view details” empty state.

## 2.2 Top Navigation Bar

The top navigation bar is common for all PMS technical modules and appears above the Components screen.

### 2.2.1 Items

From left to right (as per screenshot):

- Technical

- Dashboard

- Cert. & Surveys

- Defects

- PMS (highlighted when Components module is active)

- Admin

### 2.2.2 Behaviour

- Clicking PMS keeps user within PMS modules; Components is one of them.

- The highlight (blue background) indicates the current top-level tab.

- Switching to another tab (e.g. Dashboard, Defects) navigates away from Components and loads that module’s screen.

(No additional assumptions are made here about routing or state preservation.)

## 2.3 Left Navigation Sidebar (Module-Level)

A vertical blue sidebar is present on the extreme left. It holds icons for the main PMS modules.

### 2.3.1 Icons (top to bottom)

- Dashboard

- Components (highlighted when in this screen)

- Work Orders

- Running Hrs

- Spares

- Stores

- Reports

- Modify PMS

- Admin

### 2.3.2 Behaviour

- Clicking an icon switches module while staying inside PMS context.

- Components icon is highlighted (different background/indicator) when this screen is active.

- Behaviour of other module icons is defined in their respective module specs.

## 2.4 Main Content Area – High Level

Inside the white area to the right of the blue sidebar:

- A page title:

  - Components

- A filter bar directly below the title (Vessel / Critical Item / Search).

- A two-column layout:

  - Left: Component Tree

  - Right: Component Detail Panel

When no component is selected, the right panel shows a centered text:

“Select a component to view details”

When a component is selected, the right panel shows Sections A–H (Component Information, Running Hours, Jobs, etc.) in stacked, collapsible cards.

## 2.5 Filter Bar (Top of Components Screen)

The filter bar sits below the “Components” heading and above the tree/detail columns.

### 2.5.1 Elements

From left to right:

- Vessel Dropdown

  - Label shown as Vessel:

  - Current selection sample: V001 – MV SEAFARER

  - Dropdown options as seen in screenshot:

    - V001 – MV SEAFARER

    - V002 – MV VOYAGER

    - V003 – MV EXPLORER

- Critical Item Filter

  - Label: Critical Item:

  - Dropdown currently showing All Items

  - As per your latest instruction, this should eventually become a tick box (e.g., “Critical only”), but in the current UI we keep it as dropdown until changed.

- Search Box

  - Placeholder: Search Components...

  - Single-line text input.

- Add / Edit Component Button (aligned to the right edge of the screen)

  - Blue button: + Add / Edit Component

### 2.5.2 Behaviour

(a) Vessel Dropdown

- Selecting a vessel from this dropdown:

  - Reloads the entire component tree in the left pane for the selected vessel.

  - Reloads the right pane with:

    - A blank state (Select a component to view details) until a component is picked.

  - All data displayed (jobs, spares, history, manuals, class data) is always for the currently selected vessel.

(b) Critical Item Filter

- Current design shows a dropdown with at least:

  - All Items

  - Critical Items (for critical components only)

- When Critical Items is active:

  - Component tree should show only components marked as Critical = Yes, or highlight/filter accordingly (exact tree behaviour can be detailed later if you wish).

- You have suggested that this may later become a tickbox (“Critical”) instead of dropdown; this will be handled when UI is updated.

(c) Search Box – Smart Search

- User can type free text.

- Search must be smart and search across at least:

  - Component Name

  - SFI / Component Code

  - Fleet Equipment Code (if displayed anywhere in tree)

  - Maker

  - Serial Number

- Matching components:

  - Are highlighted in the component tree, OR

  - The tree auto-scrolls to and expands the branch where the match exists (exact visual behaviour can be defined later, but logic is: locate components by any of these keys).

(d) Add / Edit Component Button

- Behaviour depends on user role (detailed later in role section, but UI behaviour here):

  - When clicked, opens a Component Master Form for the position in the tree that is currently selected.

  - If no node is selected, system may prompt user to select where to add (tree behaviour defined in next sections).

  - The form contains sections A–H with empty fields (for new component) or prefilled fields (for editing), respecting permissions:

    - Ship users: can fill data for new components but changes go through Modify PMS workflow for approval before they appear in live tree.

    - Software provider and office admin: can commit changes directly.

(Detailed approval logic will come in later sections.)

## 2.6 Left Pane – Component Tree

The left pane has a blue header bar with label:

- COMPONENTS (all caps, white text on blue background)

Below that, a vertical tree grid is shown.

### 2.6.1 Top-level Nodes

As per SFI groups shown in screenshot:

- 1 Ship General

- 2 Hull

- 3 Equipment for Cargo

- 4 Ship's Equipment

- 5 Equipment for Crew & Passengers

- 6 Machinery Main Components

- 7 Systems for Machinery Main Components

- 8 Ship Common Systems

Each top-level row has:

- A disclosure arrow (> / caret) on the left for expand/collapse.

- When a node is selected, its background becomes light blue.

### 2.6.2 Sub-level Nodes

Example shown:

- 3 Equipment for Cargo

  - 33 DECK CRANES FOR CARGO

    - 331 ROTATING CRANES W/CRANE PILLARS

      - 331.831 Rotating single crane

      - 331.847 Rotating single crane

      - etc.

Characteristics:

- Hierarchical indentation for each child level.

- Each child node also has a disclosure arrow if it contains further children.

- The selected leaf-level component (e.g., 331.831 Rotating single crane) is highlighted.

### 2.6.3 Behaviour

- Clicking a row:

  - Sets it as selected.

  - Loads component details in right pane if the node represents an actual equipment/component (not just a grouping).

- The tree is always filtered to the currently selected vessel from the filter bar.

- No “hidden components” concept – all valid nodes are visible.

### 2.6.4 Adding Components via Tree

When “+ Add / Edit Component” is clicked while a node is selected:

- If the selected node is a 3-digit SFI group (e.g. 331):

  - System calculates the next available 3-digit code at that level (e.g. 332, 333), following the existing SFI structure for that vessel.

- If the selected node is a deeper sub-code (e.g. 331.831):

  - System calculates the next available child code under that node (e.g. 331.832, 331.833 etc., or equivalent per SFI rules).

- No dashes are allowed; only digits and dots, following whatever codes are already present from the Admin-uploaded Excel.

Actual persistence and approval will be defined under Modify PMS / Admin sections later.

## 2.7 Right Pane – Empty State

When the screen loads or when the user has not yet selected a component:

- The right pane displays a centered message:

“Select a component to view details”

- No sections (A–H) are visible in this state.

- Once a component is selected in the tree, this message disappears and the Component Detail Layout is shown.

## 2.8 Right Pane – Detail State (High Level)

When a component (leaf node or equipment node) is selected in the tree:

- The right pane is filled with a vertical stack of collapsible panels, representing Sections A to H:

  - A. Component Information

  - B. Running Hours & Condition Monitoring

  - C. Jobs

  - D. Maintenance History

  - E. Spares

  - F. Drawings & Manuals

  - G. Classification & Regulatory Data

  - H. Requisitions

- Each section has:

  - A header bar (with section letter and title).

  - Content area under it (tables and fields).

  - Collapse/expand icon (chevron) on right.

### 2.8.1 Collapsing Behaviour

- Sections can be collapsed individually.

- Even when collapsed, at least a small preview row or two may remain visible (especially for list sections like Jobs and Spares) so that the user can identify which section holds what without too much expanding/collapsing.

- Section H (Requisitions) is currently collapsed by default and acts as a placeholder for future purchase integration.

(Exact section-by-section contents and field behaviours will be defined in later sections A–H.)

## 2.9 Scroll Behaviour

Because the right pane contains multiple sections and potentially large tables:

- The right pane is vertically scrollable.

- The top filter bar and left component tree remain fixed in place when scrolling, so the user does not lose context of the selected vessel and component.

## 2.10 Responsive Behaviour (High-Level)

From the screenshots, the design is optimized for desktop usage:

- Two-column layout is maintained.

- Tables and forms are wide, suggesting a minimum horizontal resolution.

If mobile or tablet behaviour is needed, that can be separately specified; for now, this spec focuses on the desktop/web app layout as in the screenshots.

# SECTION 3 — A. COMPONENT INFORMATION

This is the primary information block for every equipment item in the PMS and is displayed as Section A when a component is selected in the component tree.

It is the first and most critical section, as all other sections (Running Hours, Jobs, Spares, Manuals, Class, History) depend on the data stored here.

## 3.1 Section Layout

- Displayed as a collapsible card titled:
A. Component Information

- Always expanded by default when a component is selected.

- Information is presented in a multi-column grid with labelled fields.

- Fields are grouped logically (Maker/Model details, Codes, Dates, Flags, Notes).

## 3.2 Fields & Behaviour (Field-by-Field Logic)

Below is a complete list of all fields exactly as seen in your screenshot, with their correct meanings, permissions, data sources, and behaviour.

### A. IDENTIFICATION FIELDS

## 1. Fleet Equipment Code

Purpose
A unique fleet-wide identifier for this equipment type.

Logic

- Assigned only by PMS Software Provider.

- Typically 10 characters (e.g., “601.001.AA”), but no strict limit enforced.

- Generated based on Maker Code + Model Code.

- All vessels using the same equipment model share this Fleet Equipment Code.

Editable by:

- Software Provider only

- Ship users → NO

- Company office → NO

Source:

- Entered via Admin Excel upload

- Or via PMS Admin interface when creating equipment master

## 2. Fleet Equipment Name

Purpose
The common fleet-wide name associated with the Fleet Equipment Code.

Logic:

- Same name across fleet for same equipment type

- Ensures consistency in jobs, spares, manuals

Editable by:

- Software Provider only

- Not editable by ship/company

## 3. Parent Component Code

Purpose
Defines the SFI-based parent of this component to maintain the hierarchical navigation structure.

Logic:

- Reflects the SFI grouping above this item

- Determines where it appears in the component tree

- Also determines code auto-generation path for new subcomponents

Editable by:

- Software Provider

- Office (in Admin PMS editor)

- Ship → NO (ship can request change via Modify PMS)

Source:

- From Admin Excel upload

- Or auto-generated by system when adding new components under a node

## 4. Component Code

Purpose:
The SFI-based code for this component on this specific vessel.

Logic:

- Strictly digits + dots (no dashes)

- Follows SFI structure uploaded by Admin

- When adding new subcomponents:

  - Under 331 → next 3-digit: 332, 333, etc.

  - Under 331.831 → next child: 331.832, 331.833, etc.

Editable by:

- Software Provider (Admin) only

- Office cannot change code after creation

- Ship cannot change

Generated by:

- System automatically when added under a selected SFI node

## 5. Component Name

Purpose:
Human-readable name of the equipment.

Logic:

- Vessel-specific name

- Should be clear and descriptive (e.g., “Rotating single crane”)

Editable by:

- Office (in Admin or Modify PMS approval stage)

- Software Provider

- Ship cannot edit here

## 6. Component Category

Purpose:
Internal classification to group components by functional type.

Examples:

- Ship's Structure

- Deck Machinery

- Engine Department

- Safety Equipment

- Accommodation

- Hull

- Equipment for Cargo

- Ship General

Editable by:

- Software Provider

- Office (Admin)

- Ship → cannot change

### B. MAKER & MODEL FIELDS

## 7. Maker

Purpose:
Manufacturer of the equipment.

Editable by:

- Office

- Software Provider

- Not editable by ship

Source:

- From Admin Excel template

- Can be selected from Maker master list

## 8. Maker Code

Purpose:
Short code for the manufacturer, used internally for Fleet Equipment Code generation.

Editable by:

- Software Provider only

- Ship/Office cannot change

## 9. Model

Purpose:
Model name/number of equipment.

Editable by:

- Office

- Software Provider

- Ship cannot edit

## 10. Model Code

Purpose:
Short code representing the model, used in Fleet Equipment Code.

Editable by:

- Software Provider only

## 11. Serial Number

Purpose:
Unique serial number printed on the equipment.

Editable by:

- Office

- Ship → NO

- Software Provider → YES

Source:

- From Admin Excel upload or manually entered by office

## 12. Drawing Number

Purpose:
Reference number for the equipment drawing/manual.

Editable by:

- Office

- Software Provider

- Ship cannot edit

### C. PHYSICAL DETAILS & ATTRIBUTES

## 13. Location

Purpose:
Physical location onboard the vessel where the equipment is installed.

Editable by:

- Office

- Software Provider

Examples:

- Engine Room

- Deck

- Steering Gear Room

- Pump Room

## 14. Critical (Yes/No)

Purpose:
Indicates whether this equipment is considered critical by the company.

Editable by:

- Company Superintendent / Technical Office

- Ship → NO

- Software Provider → YES

Impact:

- Job criticality

- Spare criticality

- Critical Items filter

- Reports

## 15. Condition Based (Yes/No)

Purpose:
Reserved for future condition-based maintenance implementation.

Current Behaviour:

- Always “No” in PMS

- Does NOT affect frequency or scheduling

- No CBM module implemented yet

Editable by:

- Office

- Software Provider

## 16. Commissioning Date

Purpose:
Date the equipment was commissioned on the vessel.

Logic:

- Used as fallback “Last Done Date” for jobs if no previous history exists

- Acts as baseline for new ships/components

Editable by:

- Office

- Software Provider

- Ship → NO

## 17. Rating

Purpose:
Any technical rating associated (e.g. “70 kW”, “20 bar”, etc.)

Editable by:

- Office

- Software Provider

## 18. Equip/System Department

Purpose:
Indicates which department owns the equipment.

Example values:

- Deck

- Engine

- Electrical

Editable by:

- Office

- Software Provider

## 19. Installation Date

Purpose:
Date equipment was physically installed onboard.

Editable:

- Office

- Software Provider

Used for:

- Determining new equipment

- Regulatory documentation

- Replacement events (via Modify PMS)

### D. RUNNING HOURS FIELDS

## 20. Running Hours

Purpose:
Shows the current running hours for this component.

Displayed:

- Last saved RH from Running Hours module

- Propagates delta updates from parent component

Editable by:

- Ship can update via Running Hours Module, NOT here

- Pencil icon here opens quick RH update form but still logs timestamp + delta

Important:

- If parent RH increases by +N → all children get +N (delta logic)

- If child replaced → its RH resets to 0, but future deltas still apply

## 21. IS Active (Yes/No)

Purpose:
Indicates whether component is currently active or not.

Typical cases:

- Equipment decommissioned

- Equipment removed from service

- Temporarily disabled

Editable by:

- Office

- Software Provider

Ship cannot change.

## 22. Vessel Code

Purpose:
Identifies the vessel to which this specific component belongs.

Editable:

- Not editable by ship

- Automatically populated based on vessel selection

- Defined through Admin Excel upload

## 23. IS Parent (Checkbox)

Purpose:
Indicates whether this component is a parent node in the component tree.

Logic:

- If IS Parent = YES:

  - Component will show children in tree

  - RH propagation applies downward

- If IS Parent = NO:

  - Component has no children under it

  - Only receives deltas from above

Editable:

- Software Provider only

- Office/Ship cannot change parent flag

## 24. Notes (Multi-line field)

Purpose:
Free text notes associated with component.

Editable by:

- Office

- Software Provider

Ship cannot edit (unless proposed via Modify PMS).

## 3.3 Field Permissions Summary Table

| Field | Ship | Office | Software Provider |
| --- | --- | --- | --- |
| Fleet Equipment Code | No | No | Yes |
| Fleet Equipment Name | No | No | Yes |
| Parent Component Code | No | Conditional (Admin only) | Yes |
| Component Code | No | No | Yes |
| Component Name | No | Yes (Admin/approval) | Yes |
| Maker / Maker Code | No | Maker: Yes, Code: No | Yes |
| Model / Model Code | No | Model: Yes, Code: No | Yes |
| Serial No | No | Yes | Yes |
| Drawing No | No | Yes | Yes |
| Location | No | Yes | Yes |
| Critical | No | Yes | Yes |
| Condition Based | No | Yes | Yes |
| Commissioning Date | No | Yes | Yes |
| Rating | No | Yes | Yes |
| Equip/System Dept | No | Yes | Yes |
| Installation Date | No | Yes | Yes |
| Running Hours | Only via RH Module | No direct edit | Yes |
| IS Active | No | Yes | Yes |
| Vessel Code | No | No | System |
| IS Parent | No | No | Yes |
| Notes | No | Yes | Yes |

## 3.4 Behaviour on Component Selection

When a user selects a component in the tree:

- Section A loads immediately with all fields displayed in read-only mode.

- If user has permissions (office/admin), editable fields show pencil icons.

- Data shown is specific to selected vessel.

- Any change triggers:

  - Validation

  - Audit logging

  - Possibly Modify PMS workflow (if ship proposes changes)

## 3.5 Source of Data for Section A

- Initial PMS Upload (Excel in Admin module)

- PMS Admin Interface (software provider)

- Modify PMS Module (for ship to propose changes)

- Running Hours Module (for RH updates)

- Work Orders Module (affects maintenance history but not Section A fields)

- Manual entries by office (allowed fields only)

## 3.6 Dependency Notes

Section A acts as the backbone for:

- Section B (Running Hours)
Uses RH and parent-child relationships defined here.

- Section C (Jobs)
Jobs are linked using Component Code & Fleet Equipment Code.

- Section E (Spares)
Spares are mapped to Component Code.

- Section F (Drawings & Manuals)
Uses Maker, Model, Drawing No for file classification.

- Section G (Class & Regulatory)
Equipment type affects survey requirements.

# SECTION 4 — B. RUNNING HOURS & CONDITION MONITORING

This is Section B in the Component Details screen. It appears directly below Section A and provides a summary of the running hours for the selected component, along with future provision for condition monitoring.

## 4.1 Section Layout

- Displayed as a collapsible panel labeled:
B. Running Hours & Condition Monitoring

- Expanded by default when a component is selected.

- Contains two main fields:

- Running Hours – Current

- Running Hours – Updated

- Pencil icon for editing running hours

(Condition Monitoring portion is reserved for future implementation but shown as part of the UI as per screenshot.)

## 4.2 Running Hours – Core Logic

Running Hours in PMS follow a strict parent–child structure:

### 4.2.1 Parent → Child Delta Rule (Mandatory Behaviour)

When the Running Hours of a parent component are updated (in Running Hours Module):

- System calculates:
Delta = New RH – Previous RH

- Delta value is applied to all child components, except those recently replaced.

Example:
Parent RH increases from 5000 → 5010

- Delta = +10

- All children RH get +10 added automatically:

  - Child 1: 2000 → 2010

  - Child 2: 0 → 10 (newly replaced component)

  - Child 3: 105 → 115

This propagation ensures correct scheduling for RH-based maintenance jobs.

### 4.2.2 Replacement Scenario

If a child component has been replaced (Running Hours reset to 0):

- After reset: RH = 0

- Next parent delta still applies normally

- New child RH = 0 + delta

This allows:

- Parent engine at 10,000 hrs

- New liner at 0 hrs

- But future running patterns still flow correctly

(This behaviour is essential for engines, compressors, pumps, hydraulic systems, rotating cranes etc.)

## 4.3 Running Hours Display Fields

### 1. Current

- Shows the latest running hours value of this component.

- Sourced from:

  - RH Module updates (ship input)

  - Parent delta propagation

  - New component replacement reset

Read-only in this screen (Section B).

### 2. Updated

- Shows the timestamp or date of last RH update.

- Ensures traceability.

- This field is read-only.

### 3. Pencil Icon (Quick Update)

Even though RH should be updated in Running Hours Module, PMS allows a quick update option here:

When user clicks the pencil icon:

- Opens a small RH edit modal

- User enters new RH

- The system:

  - Validates the value

  - Calculates delta

  - Applies delta to all children

  - Logs the update with timestamp

  - Sends change to Work Order system for RH-based job checks

Permissions:

- Ship users → Allowed to update

- Office → Allowed

- Software Provider → Allowed

- But all changes are logged (user, timestamp, delta)

## 4.4 Interaction With Running Hours Module

Running Hours Module is the primary point for RH input.

Section B:

- Displays same RH values

- Updates instantly whenever RH Module updates

- Allows delta calculation from parent

- Does not maintain its own independent log

- Never overrides the logic of RH module

Section B is a mirror + quick entry point, not a separate RH tracker.

## 4.5 Interaction With Work Orders

For RH-based jobs:

### 4.5.1 Scheduling Logic

A RH-based job has:

- Frequency in hours

- Last Done Running Hours

Next Work Order should be created when:

Current RH >= Last Done RH + Frequency – Lead Time

(Lead Time is configurable in Admin Module — you told me to remind you to add this there.)

### 4.5.2 Impact of RH Change

Every RH update triggers a recheck of all jobs under this component:

- If job is calendar-based → no impact

- If job is RH-based → system checks if threshold is reached

If threshold met → Work Order is generated in WO Module.

## 4.6 Condition Monitoring (Future Implementation)

As per your instruction:

- No condition-monitoring logic is active today

- Section title includes “Condition Monitoring” for future additions

- Possible fields (not implemented):

  - Vibration

  - Temperature

  - Oil analysis

  - Pressure monitoring

  - Sensor data

For now:

- Section exists visually

- Zero functional impact in PMS

- Ship and office cannot input anything under condition monitoring

## 4.7 Permissions Summary

| Operation | Ship | Office | Software Provider |
| --- | --- | --- | --- |
| View RH | Yes | Yes | Yes |
| Update RH | Yes (via RH module or quick update) | Yes | Yes |
| Edit RH logic | No | No | Yes |
| Modify children RH | Auto | Auto | Auto |
| Condition Monitoring input | Not implemented | Not implemented | Not implemented |

## 4.8 Data Sources

Running Hours come from:

- RH Module (primary)

- Quick edit in this section (still logged as RH module update)

- Parent delta propagation

- Component replacement event (via Modify PMS)

There is no manual editing of RH values directly in Excel uploads once vessel is active, except through controlled interfaces.

## 4.9 UX Summary

- Section B shows two simple values + a pencil icon.

- Behaviour is consistent across all components.

- Update mechanism reflects entire fleet logic (parent-child).

- Ensures RH-based maintenance is always accurate.

# SECTION 5 — C. JOBS

(Job Templates, Frequencies, Maintenance Type, Work-Order Trigger Logic)

Section C displays all scheduled maintenance jobs linked to the selected component. These jobs define what work must be performed, how often, and what type of maintenance applies. This section is directly responsible for generating Work Orders and for maintaining statutory maintenance compliance.

This section is read-only for ship staff (except proposing changes through Modify PMS), but fully functional for Office and PMS Admin.

## 5.1 Layout of Section C

- Shown as a collapsible card:
C. Jobs

- Contains:

  - Add Job button (visible only to Office and PMS Admin)

  - Jobs Table displaying all jobs linked to the component

  - Pagination or scroll depending on job count (future expandable)

The table has the following columns exactly as per your screenshot:

| Column | Meaning |
| --- | --- |
| Job No. | Unique job identifier generated from Component Code + job sequence |
| Job Title | Name/description of job |
| Maintenance Type | Inspection / Overhaul / Replacement / Servicing etc. |
| Frequency | Calendar or Running Hour frequency |
| Last Done Date | Last time maintenance was completed |
| Next Due Date | Automatically calculated (calendar jobs only) |

For running hour jobs, Next Due Date is not shown — instead PMS uses running-hour triggers.

## 5.2 Job Numbering Logic

Each job has a unique identifier:

### 5.2.1 Format Example (From Screenshot)

J0B-331.831-01

J0B-331.831-02

J0B-331.831-03

### 5.2.2 Components of Job Number

- Prefix: JOB or J0B (as used in screenshot)

- Component Code (SFI-based)

- Hyphen

- Sequence number (02, 03, 04, etc.)

### 5.2.3 Sequence Generation

When adding a new job:

- PMS finds highest sequence under this component

- Generates next number automatically

- No user input required

Example:
If last is J0B-331.831-03
→ next = J0B-331.831-04

## 5.3 Job Source & Data Creation

Jobs may enter PMS by:

### 5.3.1 Admin Excel Upload (Primary Source)

The most common method. Excel contains:

- Component Code

- Fleet Equipment Code

- Job Titles

- Maintenance Types

- Frequencies

- Last Done Information

- RH/Hr-based indicators

- Lead Time (in hours or days)

These are imported in Admin module.

### 5.3.2 Manual Job Creation (“Add Job” Button)

Allowed only for:

- Office users

- PMS Software Provider

When a job is manually added, ship users will NOT see it until the Office approves it (if your workflow requires that — we can define it in Modify PMS but you haven’t yet).

### 5.3.3 Modify PMS Request (for ship-initiated changes)

Ship cannot add or edit jobs here.
They must submit a request through Modify PMS.

## 5.4 Maintenance Type

Each job has a Maintenance Type as defined in Excel template or office entry.

Valid types include (examples based on your screenshot and maritime practice):

- Inspection

- Overhaul

- Servicing

- Testing

- Cleaning

- Lubrication

- Replacement

### Important:

Ship users CANNOT change Maintenance Type here.

Office & PMS Admin may modify it through Admin or Modify PMS workflow.

## 5.5 Frequency Logic (Critical Section)

There are two frequency types only:

### 5.5.1 Calendar-Based Frequency

Examples:

- 1 Month

- 3 Months

- 6 Months

- 12 Months

- Weekly (7 days)

- Bi-weekly (14 days)

Calendar jobs show Next Due Date in the table.

### 5.5.2 Running-Hour–Based Frequency

Examples:

- 250 hours

- 500 hours

- 1000 hours

RH-based jobs do not show Next Due Date.
Instead PMS calculates:

Remaining Hours = (Last Done RH + Frequency) – Current RH

And work-order creation follows lead-time logic.

## 5.6 Last Done Date Logic

### 5.6.1 Source of Last Done Date

- Taken from Admin Excel upload

- Or if blank → defaults to Component Installation Date

- If component is newly installed → installation date = baseline

- If Work Order is completed → Last Done Date gets auto-updated

### 5.6.2 Editable?

- Ship cannot edit

- Office cannot directly edit

- PMS Admin may edit through Admin interface

- Any ship request must go through Modify PMS

## 5.7 Next Due Date Calculation (Calendar Jobs Only)

For calendar-based jobs:

Next Due Date = Last Done Date + Frequency (in months/days)

Automatically calculated by PMS.

### Important Rules

- User cannot manually override

- Ship cannot edit

- Next Due recalculates whenever:

  - Work Order is completed

  - Last Done Date changes (due to new job import or replacement)

## 5.8 Running-Hour–Based Job Scheduling

For RH-based jobs:

### Next Due Date is not displayed

(As per your strict instruction)

Instead PMS uses:

Trigger Point = Last Done RH + Frequency

And when:

Current RH >= Trigger Point – Lead Time

A Work Order is automatically generated.

### Lead Time

- Stored in Admin module

- Can be set per job frequency category

- (Example: 200 hours before due date)

- PMS checks RH updates from:

  - Running Hours Module

  - Section B quick updates

  - Parent delta propagation

## 5.9 When Does a Work Order Get Created?

Work Orders are created automatically when job criteria are met.

### 1. Calendar Job

Today >= Next Due Date – Lead Time (days)

### 2. Running-Hour Job

Current RH >= (Last Done RH + Frequency) – Lead Time (hours)

When triggered:

- Work Order appears in Work Orders Module

- Assigned to the relevant user (Chief Engineer or assigned rank)

- Ship staff cannot modify job definition, only complete Work Order

## 5.10 Table Behaviour

### Sorting

- Table should allow sorting by:

  - Job Number

  - Job Title

  - Maintenance Type

  - Last Done Date

  - Next Due Date

### Collapsing

- Even when Section C is collapsed, PMS should show at least 1–2 job rows as a preview.

### Scrolling

- If more jobs exist, table should scroll internally without collapsing other sections.

## 5.11 Add Job Button Behaviour

The “Add Job” button:

- Visible only for Office & PMS Admin

- Hidden for Ship staff

### When clicked:

- Opens “Add Job” modal

- Fields include:

  - Job Title

  - Maintenance Type (dropdown)

  - Calendar or RH-based

  - Frequency

  - Last Done Date

  - Additional Notes

### Job Code Auto-Generation:

- PMS auto-assigns next sequence number

- No manual code entry required

### After saving:

- Job appears in Job Table

- If created from ship-side request → must pass through Modify PMS for office approval

## 5.12 Permissions

| Action | Ship | Office | PMS Admin |
| --- | --- | --- | --- |
| View job list | Yes | Yes | Yes |
| Add job | No | Yes | Yes |
| Edit job | No (via Modify PMS only) | Yes | Yes |
| Delete job | No | Yes | Yes |
| Modify frequency | No | Yes | Yes |
| Modify maintenance type | No | Yes | Yes |
| Modify last done date | No | No (except via WO completion) | Yes |
| Trigger WO | No (auto) | Auto | Auto |

Ship views only.
All edit capabilities are controlled by office/admin.

## 5.13 Data Sources

Jobs come from:

- Admin Excel Upload (main source)

- Manual job creation (office)

- Modify PMS (ship proposes, office approves)

- Fleet Equipment Code template (future scalable behaviour)

- Component Code mapping

# SECTION 6 — D. MAINTENANCE HISTORY

(Work Order Completion Records — Immutable History)

Section D shows the historical maintenance record for the selected component.
This history is automatically generated from Work Orders — never manually edited.

This section ensures PMS maintains a real, auditable, tamper-proof record of all maintenance performed on a vessel.

## 6.1 Section Layout

Displayed as a collapsible panel:

D. Maintenance History

The panel contains a table with the following columns (exactly as seen in your screenshot):

| Column | Meaning |
| --- | --- |
| Title | Work Order title (taken from job title) |
| Work Order No | Unique WO identifier |
| Assigned To | Person/Rank who completed the job |
| Status | Status of the work order (Completed/Approved) |
| Date Completed | Final approval date |

If no maintenance history exists, the table appears empty.

## 6.2 Purpose of Maintenance History

Section D provides:

- A complete audit of all maintenance ever performed on this equipment

- Statutory traceability for inspections, overhauls, and compliance

- Direct evidence for:

  - PSC inspections

  - Class surveys

  - Vetting inspections

  - Accident investigations

  - OEM warranty claims

This section is a read-only log for all users.

## 6.3 Source of Maintenance History

Maintenance History comes only from completed and approved Work Orders in the Work Orders Module.

### Flow:

- Job is triggered (calendar or RH-based).

- Work Order is generated.

- Assigned engineer completes the Work Order.

- WO goes for approval (usually Chief Engineer → Office).

- When the WO is approved:

  - Last Done Date (in Jobs Section C) updates

  - Maintenance History (Section D) gets a new entry

  - Work Order becomes immutable

## 6.4 When is a History Record Created?

A new entry is inserted into Section D only after:

- Work Order is submitted, verified, and approved.

Until approval, nothing appears in Maintenance History.

## 6.5 Fields Explained in Detail

### 1. Title

- The job title copied from the corresponding job definition.

- Example: “Crane Wire Rope Inspection”

### 2. Work Order No

- Unique ID generated by PMS when WO is created.

- Format depends on WO module spec (not assumed here).

### 3. Assigned To

- The person/rank who executed/completed the WO.

- Example: “2/E”, “Fitter”, “Motorman”

### 4. Status

Possible values:

- Approved (final and locked)

- (Work Orders in any other state are NOT shown here)

### 5. Date Completed

- Date on which Work Order received final approval.

- NOT the date when user submitted the Work Order.

## 6.6 Data Integrity Rules (Critical)

### Rule 1: Immutability

Once a maintenance history entry is created:

- It cannot be edited

- It cannot be deleted

- Not even by PMS Admin

This ensures PMS passes all audits.

### Rule 2: No Manual Entries

Ship staff or office cannot add history manually in Section D.

All entries must originate from Work Orders.

### Rule 3: RH-based logic sync

For RH-based jobs:

- When WO is approved:

  - Last Done RH updates (in Jobs Section C)

  - History entry is added

  - Remaining hours reset

### Rule 4: Calendar-based next-due sync

When WO is approved:

- Next Due Date recalculates

- History entry logs correct date

### Rule 5: Component Replacement

If component is replaced (via Modify PMS workflow):

- PMS should:

  - Close out existing WO cycles

  - Reset RH for the component

  - Create a special history entry:
“COMPONENT REPLACED”
(This behaviour can be finalized later if needed.)

## 6.7 Table Behaviour

- Table expands vertically as history grows.

- Oldest entries appear at bottom; newest at top (unless you want reverse — will follow your preference).

- Sorting should ideally be available by Date or WO Number.

- Pagination optional based on volume (we can define later).

## 6.8 Permissions Summary

| Action | Ship | Office | PMS Admin |
| --- | --- | --- | --- |
| View history | Yes | Yes | Yes |
| Add history | No | No | No |
| Edit history | No | No | No |
| Delete history | No | No | No |

Only Work Order approvals create history.

## 6.9 Dependencies

Maintenance History depends on:

- Jobs (Section C)

- Work Orders Module

- Running Hours Module

- Modify PMS (for replacement events)

Section D is completely independent of direct manual user input.

# SECTION 7 — E. SPARES

(Vessel-Specific Spare Parts Linked to Each Component)

Section E shows all spare parts that are linked to the selected component for the selected vessel.
This section is important because spares are directly used for:

- Maintenance planning

- Inventory control

- Issue/consume operations

- Reorder logic

- Critical spares tracking

- Regulatory and SFI-aligned spare grouping

- IHM (Inventory of Hazardous Materials) compliance

This specification includes only real behaviour as per your instructions — no assumptions.

## 7.1 Section Layout

Displayed as a collapsible panel titled:
E. Spares

The content is in tabular form with the exact columns visible in your screenshot:

| Column | Meaning |
| --- | --- |
| Part Code | Unique spare identifier |
| Part Name | Name/description of the part |
| Critical | Whether spare is critical (Yes/No) |
| ROB | Remaining On Board (current stock quantity) |
| Min | Minimum required stock |
| Stock | Actual recorded stock (if separate from ROB) |
| Location | Storage location onboard |
| IHM | IHM classification (Yes/No) |

If no spares are linked to this component, the table appears empty.

## 7.2 Purpose of Spares Section

This section allows users to see all spare parts required for the equipment, including:

- Critical spare parts

- Stock availability

- Minimum stock requirements

- Locations onboard

- Whether the spare is IHM regulated

This supports:

- Work Order execution

- Inventory planning

- Critical spare compliance

- PSC/Class inspections

- Preparation for drydock

- Procurement planning (future integration)

## 7.3 Data Source for Spares

All spares are populated from the Admin Excel Upload (Spare Parts Master template) and from the Spares/Stores modules.

For each spare part:

- A link exists between Part Code and Component Code.

- Only spares linked to this specific vessel are shown here.

### 7.3.1 Levels of Spare Data (as you defined):

- Vessel Level → Shown here (Section E)

- Fleet Level → Not shown

- Company Level → Not shown

(Fleet/company templates may exist but are not displayed in this component-level view.)

## 7.4 Field-by-Field Behaviour

### 1. Part Code

- Unique identifier for the spare.

- Comes from Admin Excel sheet.

- Clicking Part Code (future enhancement) may open spare details — not implemented yet.

Editable by:

- PMS Admin only

- Ship cannot modify

- Office cannot modify here

### 2. Part Name

- Human-readable spare name.

- Vessel-specific description.

Editable by:

- PMS Admin

- Office (depending on admin template permissions)

- Ship cannot edit here

### 3. Critical (Yes/No)

Represents whether the spare is a critical spare, separate from component criticality.

Important point (from you):

- A non-critical component may have critical spare(s)

- A critical component may have non-critical spares

Editable by:

- Office

- PMS Admin
Ship cannot change the criticality.

### 4. ROB (Remaining On Board)

Represents the actual available quantity of the spare onboard.

ROB is updated via:

- Stores Module

- Issue/Consume actions

- GRNs (future procurement module integration)

Displayed here only for reference.

Ship cannot edit ROB in this section.

### 5. Min (Minimum Required Stock)

Represents the minimum quantity required for compliance or safe operation.

Editable by:

- Office

- PMS Admin
Ship cannot modify.

Used for reorder calculations in Stores/Purchases (future).

### 6. Stock

If PMS distinguishes between ROB and Stock (system-specific behaviour from your screenshot):

- ROB = physically available

- Stock = accounted inventory quantity

We follow your UI screenshot exactly:

- Display Stock as a separate column

- Keeps ROB and Stock fields independent

Editable only via inventory modules, not here.

### 7. Location

Storage location onboard (e.g., Engine Store Room, Deck Store, Workshop Cabinet).

Editable by:

- Office

- PMS Admin

Ship cannot edit here; any updates must go through Stores/Inventory module.

### 8. IHM (Yes/No)

Indicates whether the spare is listed under IHM (hazardous materials).

Editable by:

- Office

- PMS Admin

Not editable by ship in this panel.

## 7.5 Add/Edit Spare (Not in this Screen)

Important rule (per your instruction):

👉 Ship user CANNOT add or modify spare linking here.
👉 Office user CANNOT add or modify spare linking here.
👉 PMS Admin may update linking only in Admin module.

The “Add Job” equivalent does NOT exist in Section E.
All spare linking must happen in the Admin Upload templates or later via dedicated Spares UI.

## 7.6 Table Behaviour

### Sorting

Table should support sorting by:

- Part Code

- Part Name

- Critical

- ROB

- Min

- Stock

- Location

### Scrolling

If spares exceed panel height, table scrolls internally.

### Collapsed View Preview

When Section E is collapsed:

- Show 1 preview row (Part Code + Part Name)

- Helps users identify that spares exist without expanding fully

## 7.7 Permissions Summary

| Action | Ship | Office | PMS Admin |
| --- | --- | --- | --- |
| View spares | Yes | Yes | Yes |
| Add spare | No | No | Yes (Admin Module) |
| Edit spare | No | No | Yes |
| Modify ROB | No | No | Through Inventory Module |
| Change linking | No | No | Yes |
| View IHM | Yes | Yes | Yes |
| Modify IHM | No | Yes | Yes |

## 7.8 Interactions With Other Modules

### 1. Work Orders

- Spare list is shown during WO execution

- Ship can select spare consumption in WO → affects ROB elsewhere

- Section E remains read-only

### 2. Stores / Spares Module

- All ROB and Stock values originate from this module

- Section E only shows data

### 3. Inventory Audit Trail

- Adjustments happen in Inventory module

- Reflected passively here

### 4. Modify PMS

- Ship may propose spare changes but cannot edit here

- Office reviews in Modify PMS

## 7.9 Data Integrity Rules

- No manual editing of spares here

- No addition/deletion from Section E

- All changes controlled by Admin or Inventory submodules

- Only spares belonging to selected vessel appear

- No merging from fleet/company templates

# SECTION 8 — F. DRAWINGS & MANUALS

(Centralized Document Storage — Component-Specific Technical Files)

Section F provides all technical documents associated with the selected component.
These include equipment manuals, drawings, catalogues, OEM documents, datasheets, installation diagrams, maintenance booklets, and other vessel-specific files.

These documents help ship staff and office teams reference the correct technical material when:

- Performing maintenance

- Preparing for drydock

- Completing Work Orders

- Assisting class/PSC inspections

- Ordering spares

- Verifying technical parameters

The section’s behaviour must follow your instructions exactly:

- Ship can VIEW

- Office can VIEW & DOWNLOAD

- Ship CANNOT download

- Files are stored centrally

- Linked via Component Code & Fleet Equipment Code

## 8.1 Section Layout

Displayed as a collapsible card:

F. Drawings & Manuals

Inside the panel:

A file list table with columns (format adjusted to match your UI):

| Column | Meaning |
| --- | --- |
| File Name | The name of the manual/drawing |
| Type | Manual / Drawing / OEM Doc / Catalogue / etc. |
| Version | Optional (if versioning is enabled later) |
| Uploaded By | User who uploaded |
| Uploaded On | Date of upload |
| Actions | “View” icon (for all), “Download” icon (office/admin only) |

Initially, the table may be empty for components without documentation.

## 8.2 Purpose of Drawings & Manuals Section

This section serves three key purposes:

- Access to all equipment documentation from anywhere in the system.

- Ensures the same manuals/drawings appear in:

  - Work Order module (during job execution)

  - Reports (if needed)

- Reduces dependency on ship’s scattered PDF files by centralizing documents in PMS Document Library.

## 8.3 Data Sources

Files are not uploaded through Section F itself.
They originate from:

### 1. Admin Excel Upload Package (initial PMS onboarding)

Some manuals/drawings may be included in onboarding packages.

### 2. PMS Admin Document Upload Interface

Software Provider can upload all manuals centrally:

- Organised by Component Code

- Fleet Equipment Code

- Maker / Model categories

### 3. Office Upload Interface

Office may upload:

- Custom drawings

- Vessel-specific modifications

- Updated manuals

- OEM files

### 4. Modify PMS

Ship can request:

- “Add New Manual”

- “Add Updated Drawing”

Office approves → file then appears in Section F.

## 8.4 View-only Behaviour (Ship)

Ship user permissions:

- Ship staff CAN view file directly in embedded PDF viewer.

- Ship staff CANNOT download the file.

- No right-click/save permitted (ideal behaviour, depends on browser restrictions).

- No upload/delete allowed.

This is essential because ships often operate under:

- OEM copyright restrictions

- Document confidentiality controls

- Vetting / Oil Major compliance requirements

Viewer behavior:

- When user clicks “View” icon → opens PDF viewer modal

- PDF viewer supports:

  - Zoom

  - Scroll

  - Page navigation

  - Fullscreen toggle

No additional tools.

## 8.5 Office/Admin Behaviour

Office and PMS Admin have more control.

### Allowed:

- View

- Download

- Replace file

- Add new file

- Delete file

### Download restriction:

- Allowed only for Office and PMS Admin

- Download button must not appear for ship staff

### File Replacement Process:

Replacing a file must preserve:

- Upload history

- Versioning (if implemented)

Simple behaviour (current):

- Office/Admin deletes old file

- Uploads new file

- System records new timestamp

## 8.6 File Types Allowed

By design, the section must support:

- PDF (primary format for manuals)

- JPEG, PNG (if diagrams are images)

- DOCX (Office may upload, system should convert to PDF if possible in future)

- XLSX (only if needed for component-specific data; viewer may not load these)

But for ship users:

- Viewer is always PDF viewer

- Non-PDF files must open via download (office only)

- Ship CANNOT open non-PDF file unless converted

This maintains safety and consistency.

## 8.7 File Linking Rules

Each file is linked to a component via:

### Primary Keys

- Component Code

- Vessel Code

### Optional Cross-Link

- Fleet Equipment Code (FEC)

Linking via FEC allows:

- Automatic preloading of manuals across vessels with identical equipment

- Vessel-specific documents override fleet documents if needed

## 8.8 UI Behaviour

### When Section F is Expanded:

- Full file list shown.

### When Section F is Collapsed:

- Show small preview chip:
e.g., “3 manuals attached”

- This ensures user knows documentation exists.

### Table Behaviour

- Sortable by File Name and Date

- Icons must be intuitive:

  - Eye icon = View

  - Download icon = Admin/Office only

If you want consistent icons, I can specify exact icon library later.

## 8.9 Upload & Delete Actions (Permissions)

| Action | Ship | Office | PMS Admin |
| --- | --- | --- | --- |
| View file | ✔️ | ✔️ | ✔️ |
| View PDF in viewer | ✔️ | ✔️ | ✔️ |
| Download file | ❌ | ✔️ | ✔️ |
| Upload file | ❌ | ✔️ | ✔️ |
| Delete file | ❌ | ✔️ | ✔️ |
| Replace file | ❌ | ✔️ | ✔️ |

## 8.10 Data Integrity Rules

- Files must be stored centrally, not vessel-side.

- Deleting a file must log:

  - Who deleted

  - Timestamp

  - File Name

  - Component Code

- Ship cannot lose access to essential documentation due to deletion errors — hence:

  - PMS Admin should confirm deletions

  - Versioning may be enabled in future

- Files must remain searchable by:

  - File Name

  - Component Code

  - Fleet Equipment Code

  - Maker

  - Model

## 8.11 Interaction With Other Modules

### 1. Work Orders

During job execution, ship staff can:

- Open manuals directly from Section F

- Viewer must open in WO module as well

### 2. Modify PMS

Ship can request:

- "Upload New Manual"

- "Add Document"

Office approves → file appears in Section F.

### 3. Reports

Future:

- “Documentation Compliance Report” may use Section F data.

# SECTION 9 — G. CLASSIFICATION & REGULATORY DATA

(Component-Level Class Surveys, Certificates & Regulatory Compliance)

Section G displays all class-related and regulatory compliance requirements associated with the selected component.
These may include:

- Class survey requirements (Annual, Intermediate, Renewal, Special, Damage)

- OEM certification

- Statutory compliance items

- Company-specific requirements

- Equipment test requirements

It is critical because inspectors and auditors rely heavily on this section.

## 9.1 Section Layout

Shown as a collapsible panel:

G. Classification & Regulatory Data

Inside the panel is a table with columns (exactly as seen in screenshot):

| Column | Meaning |
| --- | --- |
| Classification Society | Example: DNV, ABS, LR, IRS |
| Certificate No. | Certificate reference number |
| Last Class Survey | Date last class survey was completed |
| Next Class Survey | Date next survey is due |
| Survey Type | Type of survey (Annual, 5-year, Intermediate, etc.) |
| Class Requirements | Text or reference of requirement |
| Class Code | Internal class code (optional) |
| Information | Notes / remarks |

Empty state shows blank table.

## 9.2 Purpose of This Section

To record:

- All class-required actions for this equipment

- All certificate details

- Past survey and next survey due dates

- Inspection/test requirements

- Company regulatory requirements

- Any additional compliance notes

This section supports Ship, Office, Class Surveyors, and Auditors.

## 9.3 Multi-Row Survey Support (CRITICAL)

You confirmed:
👉 Each component may have multiple surveys.

This is essential for:

- Cranes → Annual + 5-year + Load Test

- Main Engines → Multiple cylinder inspections, annual performance tests

- Boilers → Internal & external inspections

- Safety equipment → Multiple statutory cycles

Therefore, PMS must support multiple rows per component.

Each row is a separate class/regulatory requirement.

## 9.4 Data Source

Class data may come from:

### 9.4.1 Admin Excel Upload

During initial PMS onboarding:

- Class survey details

- Certificate numbers

- Survey types

- Historical dates

### 9.4.2 Manual Entry by Office

Office staff (Superintendent / Marine / Technical) may add or update:

- Survey dates

- Requirements

- Information field

### 9.4.3 Modify PMS (Future)

Ship may propose:

- Updates to Class data

- Fixes to incorrect survey dates

Office must approve before it updates Section G.

### 9.4.4 Certificate Upload (linked to Section F)

Related certificates can be uploaded in Drawings & Manuals.

## 9.5 Field-by-Field Logic

Below is the precise description of each column.

### 1. Classification Society

Examples:

- DNV

- ABS

- Lloyd’s Register

- ClassNK

- RINA

- IRS

Editable only by:

- Office

- PMS Admin

Ship cannot modify.

### 2. Certificate No.

- Exact certificate reference number for that survey/inspection.

- May be uploaded from initial Excel or entered manually.

Editable by:

- Office

- PMS Admin

Ship cannot edit.

### 3. Last Class Survey

Date of last survey performed.

Logic:

- Editable by Office

- Set automatically when survey Work Order (future) is approved

- Ship cannot edit

If empty, it may mean:

- New equipment

- No previous survey data

- Data not yet entered by office

### 4. Next Class Survey

Expected due date of next survey.

Logic:

- Office enters this manually

- May be calculated by applying survey rules (if you add this functionality later)

- Ship cannot modify

This field is required for:

- Survey tracking

- Vetting

- PSC/Class inspections

### 5. Survey Type

Examples:

- Annual Survey

- 5-Year Survey

- Intermediate Survey

- Damage Survey

- OEM Test

- Statutory Requirement

- Internal Company Requirement

Editable by:

- Office

- PMS Admin
Ship cannot change.

### 6. Class Requirements

Text describing requirement for this component.

Examples:

- “Annual load test required as per DNV rules.”

- “5-year overhaul mandatory.”

- “Hydrostatic test required.”

- “Wire rope renewal every 5 years.”

Editable by:

- Office

- PMS Admin

### 7. Class Code

Optional internal code issued by classification society.

Used for:

- Identification

- Mapping to regulatory documents

Editable only by Office and Admin.

### 8. Information

Free text notes.

Examples:

- “Survey postponed due to port restrictions.”

- “OEM inspector attended.”

- “Next survey aligned with drydock schedule.”

Editable by:

- Office

- PMS Admin
Ship cannot edit.

## 9.6 Add/Edit Survey Row

Buttons/Actions:

### Office:

- Can add new row

- Can edit existing rows

- Can delete row (with audit trail)

### Ship:

- Cannot add/edit/delete

- Can only see the list

If ship wants to modify survey data:
👉 Must raise change request through Modify PMS.

## 9.7 Table Behaviour

### Sorting

Supported on:

- Survey Type

- Last Survey

- Next Survey

- Classification Society

### Collapsing

If Section G is collapsed:

- Show a preview (“2 Class Surveys”, “3 Regulatory Items”)

- Helps users understand there is data inside

## 9.8 Data Integrity Rules

- No manual changes by ship.

- Office must maintain accurate survey data.

- Each update must be logged with:

  - User

  - Timestamp

  - Before/after values

- Filing errors can only be corrected by office/admin.

- If certificates are attached in Section F, cross-reference can be shown in Section G (optional future enhancement).

## 9.9 Interaction With Other Modules

### 1. Drawings & Manuals (Section F)

Class certificates uploaded there.

### 2. Work Orders (Future Enhancement)

If you later implement “Survey WO”:

- WO approval updates “Last Class Survey” automatically

- And recalculates “Next Class Survey”

### 3. Modify PMS

Ship requests changes, office approves.

# SECTION 10 — H. REQUISITIONS

(Placeholder for Future Purchase/Stores Integration — Collapsed by Default)

Section H represents the future area where all purchase requisitions linked to the selected component will appear.

This includes:

- Spare part requisitions

- Service requisitions

- Repair requisitions

- Consumable requests

However, as per your direct instruction, the functionality must not be implemented now, and the panel must remain collapsed.

## 10.1 Section Layout

Displayed as a collapsible panel:

H. Requisitions

### Current Behaviour (Mandatory):

- The panel remains collapsed by default.

- The contents inside the panel should NOT be implemented.

- Only the panel header is visible.

This respects your requirement:

“just collapse the pane.”

## 10.2 Table Design (For Future Use — Do Not Implement Functionality)

The table shown in your screenshot contains columns:

| Column | Purpose |
| --- | --- |
| Req. ID | Unique requisition identifier |
| Req. Date | Date requisition was created |
| Title / Description | Summary of requested item or service |
| Requested Date | Date the item/service is needed |
| Status | Draft / Submitted / Approved / Ordered (future) |
| Remarks | Free text comment |

### Important:

- These columns must exist only visually, for layout reference if developer needs to style UI.

- But all logic behind them is disabled.

## 10.3 Behavioural Rules (As per your instruction)

### 1. User Interaction

- Ship: Cannot click anything, cannot add requisitions.

- Office: Cannot add/edit requisitions yet.

- PMS Admin: Should not add/edit anything from this panel.

### 2. No Data Loading

- Table should remain empty.

- No backend calls or data retrieval.

### 3. No Buttons

- No “Add Requisition” or “Edit” button should exist.

### 4. Fully Collapsed

- When the user expands the panel manually:

  - Show the empty table for layout reference only.

  - But keep all rows non-interactive.

  - No selection allowed.

### 5. No Links to other Modules

- No linking to Stores, Purchases, or Inventory yet.

- No pop-ups, no forms, no redirects.

## 10.4 Future Scope (Document Only — No Implementation Now)

If developed later, Section H will eventually support:

- Creating component-linked requisitions

- Auto-filling part codes

- Workflow for approval

- Linking to purchase orders

- Linking to inventory receipts

- Vendor selection

- Integration with IMPA codes (if added)

However, none of these should be implemented now.

## 10.5 Permissions Summary

| Action | Ship | Office | PMS Admin |
| --- | --- | --- | --- |
| View collapsed panel | Yes | Yes | Yes |
| Expand panel | Yes | Yes | Yes |
| Add requisition | ❌ | ❌ | ❌ |
| Edit requisition | ❌ | ❌ | ❌ |
| Delete requisition | ❌ | ❌ | ❌ |
| Trigger workflow | ❌ | ❌ | ❌ |

## 10.6 Mandatory Instruction to Developers

“Do not build any functional API, UI flow, or database calls for Section H.
Keep it as a collapsed placeholder EXACTLY as per screenshot.”

This must be respected to avoid misaligned development.

# SECTION 11 — ROLE-BASED ACCESS (COMPONENTS MODULE)

There are three major user groups in the PMS related to this module:

- Ship Users
(Master, Chief Engineer, 2/E, Electrical Officer, etc.)

- Company Office Users
(Technical Superintendent, Marine Superintendent, Crewing/HSSEQ if required)

- PMS Software Provider (Admin)
(Developers / Implementers with full control)

Each role has different levels of visibility and modification rights.
The Components Module is sensitive, as incorrect editing can corrupt the PMS.

Below is the exact access matrix based ONLY on the rules you provided.

## 11.1 SHIP USERS (ONBOARD CREW)

### Overall Access Level:

View-only for Components Module
Can update Running Hours (via RH module or quick edit)
Cannot modify structure or master data

## 11.1.1 Allowed Actions

- View entire component tree

- Expand/collapse SFI levels

- Select components and view Sections A–H

- Update Running Hours (Section B quick edit + RH module)

- View Jobs list (Section C)

- View Maintenance History (Section D)

- View Spares (Section E)

- View Manuals (Section F — PDF viewer only)

- View all Class/Regulatory rows (Section G)

- Expand Requisitions (Section H collapsed placeholder)

### Additional allowed:

- Propose changes through Modify PMS module
(e.g., adding new component, changing job frequency)

## 11.1.2 Restricted Actions (NOT allowed)

Ship users CANNOT:

- Add/edit/delete components

- Change Component Name

- Change Component Code

- Change Parent Code

- Add/edit/delete jobs

- Change job frequency or type

- Modify spares

- Add/edit manuals

- Download manuals

- Change class data

- Add/edit/delete class survey rows

- Create or edit requisitions in Section H

- Modify criticality status

- Modify IS Parent or IS Active flags

- Upload any files

- Change any technical details in Section A

## 11.1.3 Ship User Summary

Ship can view everything, update Running Hours, and propose changes,
but cannot change PMS structure or critical data.

## 11.2 COMPANY OFFICE USERS

### Overall Access Level:

Full edit access for vessel-specific data, not fleet-wide structures.

Office users include:

- Technical Superintendent

- Marine Superintendent

- HSSEQ (for certain components)

- Company Admins

## 11.2.1 Allowed Actions

### Component Information (Section A)

Office can edit:

- Component Name

- Maker

- Model

- Serial Number

- Drawing Number

- Location

- Critical (Yes/No)

- Condition Based (Yes/No)

- Commissioning Date

- Rating

- Department

- Installation Date

- IS Active (Yes/No)

- Notes

Office cannot edit:

- Fleet Equipment Code

- Maker Code

- Model Code

- Component Code

- Parent Component Code

- IS Parent flag

### Running Hours (Section B)

- Office can update Running Hours (quick edit)

- Office can view delta propagation (system-controlled)

- Office cannot edit RH logic

### Jobs (Section C)

Office can:

- Add new jobs

- Edit job title

- Modify Maintenance Type

- Change Frequency

- Change Last Done Date (only via Admin/approval)

- Delete job (with caution — triggers Modify PMS review)

Office cannot:

- Modify Fleet-wide job templates

- Modify job numbering logic

- Edit PMS structure

### Maintenance History (Section D)

Office can:

- View complete history

- Approve Work Orders (done in WO module)

Office cannot:

- Edit history

- Delete history

### Spares (Section E)

Office can:

- Edit spare criticality

- Edit spare minimum quantity

- Edit spare location (depending on Stores module behavior)

Office cannot:

- Create new spare part codes

- Modify vessel-fleet-company hierarchy

- Delete spares from PMS here

### Manuals (Section F)

Office can:

- Upload manuals

- Replace manuals

- Delete manuals

- Download manuals

Ship cannot download.

### Class/Regulatory (Section G)

Office can:

- Add survey rows

- Edit class details

- Delete survey rows

### Requisitions (Section H)

Office cannot:

- Add requisitions (future)

Panel remains collapsed.

## 11.2.2 Restricted Actions

Office cannot:

- Change any Fleet-wide master data

- Change SFI structure

- Assign or modify Fleet Equipment Code

- Modify Maker Code / Model Code

- Edit Component Code

- Delete major components without Modify PMS approval

- Modify IS Parent flag

- Change component hierarchy

- Bypass PMS Admin restrictions

## 11.2.3 Office User Summary

Office can edit vessel-specific data, manage jobs, class, spares, manuals,
but cannot modify fleet templates or core PMS architecture.

## 11.3 PMS SOFTWARE PROVIDER (ADMIN)

### Overall Access Level:

Full unrestricted access (Fleet + Vessel + Global Master Data).

This user controls:

- PMS templates

- SFI structure

- Fleet Equipment Codes

- Admin Excel Uploads

- All customization

- New fields, label editing, structure changes

## 11.3.1 Allowed Actions

PMS Admin can:

### Component Structure

- Add/edit/delete any component

- Modify Component Code

- Modify Parent Code

- Change SFI hierarchy

- Change IS Parent flag

- Change any label in the UI

- Add/remove fields

- Modify categories, departments

- Upload initial PMS via Excel templates

### Jobs

- Add/edit/delete jobs

- Modify job numbering logic

- Modify job frequency & type

- Change fleet-wide job templates

### Spares

- Add/edit/delete spares at fleet/company/vessel level

- Modify spare linking

- Set IHM linkage

### Manuals/Documents

- Upload, replace, delete

- Manage centralized document library

### Class/Regulatory

- Fully editable

- Import via Excel

- Configure class survey templates

### Running Hours

- Edit RH values

- Edit parent/child RH logic

If needed, Admin can override any values.

## 11.3.2 Restricted Actions (None)

PMS Admin has no restrictions except:

- Must maintain system integrity

- Cannot bypass audit logs

## 11.3.3 PMS Admin Summary

PMS Admin can modify everything, including SFI structure, template fields, and master data.
This is the top-level configuration authority.

## 11.4 Role Matrix Summary Table

| Action | Ship | Office | PMS Admin |
| --- | --- | --- | --- |
| View Component Tree | ✔️ | ✔️ | ✔️ |
| Add Component | ❌ | Limited (vessel only) | ✔️ |
| Edit Component | ❌ | Yes (limited) | ✔️ |
| Delete Component | ❌ | Limited via Modify PMS | ✔️ |
| Update Running Hours | ✔️ | ✔️ | ✔️ |
| Edit Running Hours Logic | ❌ | ❌ | ✔️ |
| View Jobs | ✔️ | ✔️ | ✔️ |
| Add/Edit Jobs | ❌ | ✔️ | ✔️ |
| Delete Jobs | ❌ | ✔️ | ✔️ |
| View Maintenance History | ✔️ | ✔️ | ✔️ |
| Edit History | ❌ | ❌ | ❌ |
| View Spares | ✔️ | ✔️ | ✔️ |
| Modify Spares | ❌ | Yes (vessel spares) | ✔️ |
| View Manuals | ✔️ | ✔️ | ✔️ |
| Download Manuals | ❌ | ✔️ | ✔️ |
| Upload Manuals | ❌ | ✔️ | ✔️ |
| View Class Data | ✔️ | ✔️ | ✔️ |
| Edit Class Data | ❌ | ✔️ | ✔️ |
| Requisitions (Placeholder) | View collapsed | View collapsed | View collapsed |

# SECTION 12 — DATA FLOW & BACKEND LOGIC (COMPONENTS MODULE)

(How Component Data Interacts With Running Hours, Jobs, WO, Spares, Class, Modify PMS, and Admin Uploads)

This section describes the end-to-end working logic behind the Components Module.
It defines:

- How component data is created

- How it moves through the PMS

- How it affects WO creation

- How RH propagation works

- How spare linking works

- How class data behaves

- How Modify PMS controls structural changes

This section ensures that the UI specification connects correctly with backend behaviour.

## 12.1 MASTER DATA SOURCES

Component data in PMS can originate only from three controlled entry points:

## 12.1.1 Admin Excel Upload (Primary Source)

This is the main onboarding mechanism. Admin uploads the full PMS in Excel templates:

- Component Register

- Jobs Master

- Spare Parts Master

- Maker/Model Data

- Class/Regulatory Data

- Document Index (optional)

This establishes the initial baseline.

## 12.1.2 PMS Admin Interface

The software provider can:

- Add components

- Edit core fields

- Change SFI structure

- Assign Fleet Equipment Codes

- Upload manuals

- Add/modify job templates

This is the only role that may change global PMS architecture.

## 12.1.3 Modify PMS (Ship → Office Approval Flow)

Ship proposes changes such as:

- New component

- Change of name

- Change in job frequency

- Add/remove spares

- Add new manual

Office approves → PMS updates live structure.

No change enters PMS without approval.

## 12.2 COMPONENT CREATION WORKFLOW

### 12.2.1 Admin/Office Creation

Admin creates a component by:

- Selecting a position in the SFI tree

- System determines next available code

- Admin completes Section A fields

- Component becomes active immediately

### 12.2.2 Ship-Initiated Creation

Ship follows:

- Ship clicks “Add Component”

- Fills all fields

- Submission goes to Modify PMS

- Office reviews & approves

- After approval → component appears in tree

- Jobs/spares linking happens after approval

Ship NEVER directly creates components in live PMS.

## 12.3 RUNNING HOURS BACKEND LOGIC (Critical Logic)

Running Hours are governed by the Delta Propagation Algorithm:

## 12.3.1 Parent RH Update

When parent component RH changes:

### Step 1: System reads previous RH

### Step 2: System reads new RH

### Step 3: System calculates delta

Delta = New RH – Old RH

### Step 4: For each child component

Child RH = Child RH + Delta

### Step 5: System logs:

- Delta value

- New RH

- Timestamp

- User ID

## 12.3.2 Special Rule — Replacement Event

If child has RH reset to 0 due to replacement:

When parent delta occurs:

Child RH = 0 + Delta

Replacement events must be logged in Modify PMS and Section A “Notes”.

## 12.3.3 Work Order Trigger Check

After any RH update:

System checks every RH-based job under that component:

If Current RH >= (Last Done RH + Frequency – Lead Time)

→ Create Work Order automatically

Work Order is pushed to WO module.

## 12.4 JOB FREQUENCY & WORK ORDER LOGIC

## 12.4.1 Calendar-Based Job Trigger

Next Due = Last Done Date + Frequency (e.g. 6 months)

System checks:

Today >= Next Due – Lead Time (days)

→ Create WO

Ship cannot modify schedules.

## 12.4.2 RH-Based Job Trigger

Trigger = Last Done RH + Frequency

System checks:

Current RH >= Trigger – Lead Time (hours)

→ Create WO

No Next Due Date is shown for RH jobs.

## 12.4.3 When WO is Completed

- Ship completes WO form

- Office approves

- System updates:

  - Last Done Date

  - Last Done RH

  - Maintenance History (immutable)

  - Resets RH logic for next cycle

## 12.5 MAINTENANCE HISTORY PIPELINE

Maintenance History (Section D) is never manually modified.

It is filled ONLY by Work Orders.

### Pipeline:

- Job triggers WO

- Ship executes WO

- Office approves WO

- PMS inserts new row into Maintenance History

History contains:

- Job Title

- WO No

- Assigned Person

- Status = Approved

- Date Completed

Absolutely immutable.

## 12.6 SPARES LINKING LOGIC

## 12.6.1 Spare-Part Linking Structure

Each spare is linked to a component via:

- Component Code

- Part Code

- Vessel Code

### Spares DO NOT appear fleet-wide even if FEC is same.

Only vessel-specific spares appear in Section E.

## 12.6.2 Stock Logic

Stock values come from:

- Stores Module

- Issue/Consume

- Inventory Audit

- GRN (future)

Not editable in Components Module.

## 12.6.3 Critical Spare Logic

Each spare has its own Critical (Yes/No) independent of component criticality.

But both affect reports.

## 12.7 DOCUMENT & MANUALS LOGIC

## 12.7.1 Storage Mechanism

Files stored in central PMS Document Library indexed by:

- Component Code

- Fleet Equipment Code

- Vessel Code

- File Type

Ship can VIEW only.
Office/Admin can upload/delete/download.

## 12.7.2 Viewer Behaviour

- PDF viewer loads within modal

- Ship cannot download

- Office/Admin can download

## 12.8 CLASS & REGULATORY LOGIC

## 12.8.1 Multi-Survey Rows

Each component may have:

- Annual Survey

- 5-year Survey

- Intermediate

- Damage

- OEM tests

These rows are independent.

## 12.8.2 Update Logic

Only Office/Admin can:

- Add

- Edit

- Delete

Ship can view only.

## 12.8.3 Certificate Relationship

Certificates uploaded in Section F may correspond to survey rows in Section G.

## 12.9 MODIFY PMS WORKFLOW (CRITICAL CONTROL MODULE)

Modify PMS acts as the safety gate for all structural changes.

### Ship can propose:

- New component

- Edit component name

- Change job frequency

- Add spare

- Add manual

- Change location

- Correction to wrong data

### Office MUST approve before any update becomes active.

If rejected → no change to live data.

Modify PMS ensures:

- Data integrity

- Auditability

- Controlled configuration

## 12.10 ADMIN MODULE RELATIONSHIP

Admin module controls:

- Initial upload of PMS structure

- Component Register

- Job Master

- Spare Master

- Class Data

- Maker/Model master

- Lead Time for frequency triggers

- Document type master

- Component category master

Admin is the heart of system configuration.

## 12.11 BACKEND DATA MODEL SUMMARY

Core keys:

- Component Code

- Fleet Equipment Code

- Vessel Code

- Part Code

- Job Code

- Work Order No

All relational behaviour is driven by these fields.

## 12.12 AUDIT LOGGING (MANDATORY)

Every backend update must log:

- User ID

- Action

- Old Value → New Value

- Timestamp

- IP (optional)

- Vessel Code

- Component Code

Audit log MUST exist for:

- RH changes

- Job modifications

- Spare changes

- Document uploads

- Class survey updates

- Component creation/deletion

- Modify PMS approvals

# SECTION 13 — DATABASE STRUCTURE / FIELD MAPPING (COMPONENTS MODULE)

This section defines the conceptual database structure for the Components module.
It is not a strict physical schema (column types, indexes, etc.), but a clear mapping of what tables are needed, what fields they must hold, and how they relate.

It covers only what we have explicitly defined so far for the Components module and its interactions (A–H sections + RH + Jobs + History + Spares + Manuals + Class).

## 13.1 Overview of Core Entities

For the Components module, PMS must maintain at least the following core tables/entities:

- components – Component master (Section A)

- fleet_equipment_master – Fleet Equipment definitions

- component_running_hours_log – RH updates + deltas

- component_jobs – Job definitions (Section C)

- component_maintenance_history – History (Section D)

- spares_master – Spare part master (global)

- component_spares_link – Mapping of spares to components (Section E)

- component_documents – Drawings & Manuals (Section F)

- component_class_regulatory – Class and regulatory rows (Section G)

- audit_log – Global audit logging (for all changes)

Work Orders, Inventory, Modify PMS, Admin, etc., will use their own tables but link to these.

## 13.2 components (Component Master – Section A)

Represents each equipment item onboard a specific vessel.

Table: components

Key fields:

- id – Internal primary key

- vessel_code – Vessel identifier

- component_code – SFI-based component code (per vessel)

- parent_component_code – SFI code of parent node (can also be parent_component_id if relational).

- fleet_equipment_code – Fleet-wide identifier (FEC)

- component_name

- component_category

- maker

- maker_code

- model

- model_code

- serial_number

- drawing_number

- location

- is_critical (Yes/No)

- is_condition_based (Yes/No – future use)

- commissioning_date

- rating

- department (Deck / Engine / Electrical)

- installation_date

- running_hours_current (for latest RH snapshot)

- is_active (Yes/No)

- is_parent (Yes/No)

- notes (text)

- created_by, created_at

- updated_by, updated_at

Relationships:

- 1 → many with component_jobs

- 1 → many with component_maintenance_history

- 1 → many with component_spares_link

- 1 → many with component_documents

- 1 → many with component_class_regulatory

- 1 → many with component_running_hours_log

## 13.3 fleet_equipment_master (Fleet Equipment Code Logic)

Defines fleet-level equipment types shared across vessels.

Table: fleet_equipment_master

Key fields:

- fleet_equipment_code (FEC – primary key)

- fleet_equipment_name

- maker

- maker_code

- model

- model_code

- description (optional)

- is_active

Relationships:

- 1 → many with components (same FEC used across vessels)

- May relate to job templates, spare templates, etc. (future).

## 13.4 component_running_hours_log (RH Log & Delta Tracking)

Stores every RH change for traceability.

Table: component_running_hours_log

Key fields:

- id

- vessel_code

- component_code

- previous_rh

- new_rh

- delta_rh

- updated_by (user id)

- updated_at (timestamp)

- update_source (e.g., RH_MODULE, COMPONENT_QUICK_EDIT)

This table is used for:

- RH propagation logic (parent → child)

- Audit trail

- RH-based job recalculation

components.running_hours_current should always reflect the latest RH; the log holds all history.

## 13.5 component_jobs (Jobs – Section C)

Represents all jobs linked to a component on a specific vessel.

Table: component_jobs

Key fields:

- id

- vessel_code

- component_code

- job_number (e.g., J0B-331.831-01)

- job_title

- maintenance_type (Inspection / Overhaul / etc.)

- frequency_type (CALENDAR or RUNNING_HOURS)

- frequency_value (numeric, e.g., 6, 250, 1000)

- frequency_unit (months/days/hours – as needed)

- last_done_date (for calendar jobs)

- last_done_rh (for RH jobs)

- lead_time_value (as configured in Admin)

- lead_time_unit (days/hours)

- is_active (Yes/No)

- created_by, created_at

- updated_by, updated_at

Relationships:

- Many component_jobs → 1 components

- Work Orders table will have job_id / job_number linking back here.

## 13.6 component_maintenance_history (Section D)

Stores immutable history entries for completed Work Orders.

Table: component_maintenance_history

Key fields:

- id

- vessel_code

- component_code

- job_id (FK to component_jobs)

- work_order_id (FK to WO table)

- title (job title snapshot)

- assigned_to (rank or name at the time)

- status (e.g., APPROVED)

- completed_at (final approval date)

- created_at (same as completed_at or log time)

Important rule:
No UPDATE or DELETE allowed on this table once row is created (at code level).

## 13.7 spares_master (Global Spare Definition)

Global spare definition for company/fleet/vessel.

(This table exists at system-level, but Components module reads from it.)

Table: spares_master

Key fields:

- part_code (unique)

- part_name

- description

- ihm_flag (Yes/No)

- maker (optional)

- unit_of_measure

- is_active

spares_master is used by multiple modules; Components only consumes it via link table.

## 13.8 component_spares_link (Section E)

Links spare parts to components for a specific vessel.

Table: component_spares_link

Key fields:

- id

- vessel_code

- component_code

- part_code (FK to spares_master)

- is_critical_spare (Yes/No)

- min_stock

- location (text or storage location code)

- created_at, created_by

- updated_at, updated_by

Stock/ROB normally live in Inventory/Stores tables, but Components uses ROB/Stock view joined via part_code + vessel_code.

## 13.9 Inventory / Stock Tables (Referenced Only)

Not fully defined here (separate module), but for clarity:

Example tables:

- inventory_stock

  - vessel_code

  - part_code

  - rob_quantity

  - stock_quantity

  - last_counted_at

- inventory_movements

  - movement_id

  - vessel_code

  - part_code

  - movement_type (Issue, Receive, Adjust)

  - quantity

  - work_order_id (if from WO)

  - timestamp

Section E (Spares) view = join of:
component_spares_link + spares_master + inventory_stock.

## 13.10 component_documents (Section F – Drawings & Manuals)

Stores document metadata and file references.

Table: component_documents

Key fields:

- id

- vessel_code

- component_code

- fleet_equipment_code (optional, for fleet-level documents)

- file_name

- file_type (Manual / Drawing / OEM / Certificate / etc.)

- file_path or storage_url (actual stored location)

- uploaded_by

- uploaded_at

- can_ship_view (Yes/No, generally Yes)

- can_ship_download (always No for manuals, as per current rule)

- is_active

## 13.11 component_class_regulatory (Section G)

Stores rows of class/regulatory requirements per component.

Table: component_class_regulatory

Key fields:

- id

- vessel_code

- component_code

- classification_society

- certificate_number

- last_survey_date

- next_survey_date

- survey_type

- class_requirements (text)

- class_code (optional)

- information (notes)

- created_by, created_at

- updated_by, updated_at

One component may have multiple records in this table.

## 13.12 audit_log (Global for PMS, critical for Components)

Provides the audit trail PMS must have for every sensitive change.

Table: audit_log

Key fields:

- id

- timestamp

- user_id

- vessel_code

- component_code (nullable if change was fleet-level)

- entity_type (e.g., COMPONENT, JOB, SPARE_LINK, DOCUMENT, CLASS_DATA, RUNNING_HOURS)

- entity_id (e.g., component_id, job_id)

- action_type (CREATE, UPDATE, DELETE)

- field_name

- old_value

- new_value

- source (WEB, ADMIN_UPLOAD, MODIFY_PMS, etc.)

This log is read-only and cannot be modified.

## 13.13 High-Level Relationships Summary

- components
↳ has many component_jobs
↳ has many component_running_hours_log
↳ has many component_maintenance_history
↳ has many component_spares_link
↳ has many component_documents
↳ has many component_class_regulatory

- fleet_equipment_master
↳ has many components

- spares_master
↳ has many component_spares_link

- component_jobs
↔ (many) work_orders (external module)
↔ component_maintenance_history

- component_documents
may be reused in other modules (e.g., WO view)

- audit_log
↔ referenced by all write operations

## 13.14 How A–H Sections Map to Tables

- Section A – Component Information
→ components
(+ fleet_equipment_master for shared data)

- Section B – Running Hours
→ components.running_hours_current
→ component_running_hours_log

- Section C – Jobs
→ component_jobs

- Section D – Maintenance History
→ component_maintenance_history

- Section E – Spares
→ component_spares_link + spares_master + inventory tables

- Section F – Drawings & Manuals
→ component_documents

- Section G – Class & Regulatory Data
→ component_class_regulatory

- Section H – Requisitions
→ No table required yet (future link to Purchases).